Gate Pass & Safety Requirements

Safety Terms:

      - General Safety Regulations:

- A dedicated Safety Supervisor shall be deputed at site under your scope and who shall be responsible for compliance to all Safety requirements and safe working practices for the job by your workmen.

- The cost of Safety Measures is included in the above rates for adherence to all safety precautions during the execution of the work.

- Suitable and sufficient parapet wall or railing to prevent persons from falling from the sloping roof/ side of any deep excavation area is to be provided.

- Information & warning about the danger & chances of falling from height to be provided to all the persons engaged in the work.

- You shall be responsible for taking out suitable insurance policies for the risks for all workmen, supervisors, tools, plant, equipment & building materials etc. at no extra cost.

- You shall be responsible for the safety of all employees and/or workmen employed or engaged by you on and in connection with the work and for any Incident/ Near Miss/ Accident you shall report to CESC and other local authorities concerned all cases of accidents howsoever caused and where ever occurring on the works and shall make adequate arrangements for rendering immediately all possible first aid to the victims of the accidents. A report to be submitted after proper investigation reflecting the cause and remedial measures should be taken within 7 days.

- You shall inform your employees about hazards associated with their jobs. Such information should be communicated to your employees through toolbox meetings, classroom trainings, notice board displays or through writings on Work Permit system etc. in a language understood by your employees. You shall also provide regular training to your employees regarding Safe Work Procedures to be followed at site.

- Permission for persons to work and Permission for bringing tools and equipment shall be obtained from us from us prior to commencement of work.

- All employees working under your supervision / contract must be declared medically fit.

- Work to be carried out in compliance to the site-specific SOP prepared for working under Covid19 situation.

      - Procedure for application of Gate Pass:

Requisite Enclosures:

- Two copies of recent passport size photograph of each employee /workman whose name appeared on that application duly signed by the employee himself at the back of the photo.

- One photocopy of Voters ID Card/Ration Card/Passport or any other ID acceptable by the government for the purpose has to be produced for further proceedings.

- Medical Test Certificates as mentioned above declaring fit for the work.

- ESI & PF Related documents as mentioned above.

Application procedure:

- Application duly filled in (along with the relevant documents) and addressed to Security Officer, shall be forwarded by the concerned Departmental Manager or any other manager (in absence of the concerned Manager) for onward transmission to Personnel Officer.

- If found OK, the Personnel Officer gives clearance for necessary safety training (for new gate passes or training provided more than 6 months before).

- After endorsement of Safety training by Safety Officer / PTC, Personnel Officer on compliance shall give clearance to Security Officer for gate pass.

- Security Officer arranges for issuance / renewal of Gate Pass after ensuring the above steps.

      - Work Permit:

You shall follow the permit to work system established by us to do assigned job. Refer Permit to Work Standard for respective permit system.

- Permit to work (PTW) for any kind of work:

- Contractor to take PTW for any kind of work from the concerned department before commencement of work along with the following permit appropriate for the job.

- For working at Height above 2.0 m – Double layer steel scaffoldings with tied platform Scaffolding made of following materials & dimensions. A typical sketch can be obtained from the concerned engineer.

Material:

- Healthy Steel tubes (50 mm OD x 2.5mm thk) – IS Grade for horizontal, vertical, bracing support, guard rails & access

- Healthy clamps & holders

- Base Plate - Min size 230 X 400 mm and proper resting on concrete / solid ground

SAFETY NET – Compulsory at places, to ensure that the free fall of persons working at height should not be more than 2 meters from the working platform level. Decision of Safety Officer is FINAL & binding.

- HOT PERMIT – If any job involves generation of open / visible flame –All precautions against fire hazards (like keeping fire extinguishers of all kind, water source, sand, wet gunny bags etc.)  to be taken during execution of work.

- PERMIT TO WORK AT CONFINED SPACE - If any job involves persons working in enclosed space / vessel.

All PERMITs shall be issued by the Department and forwarded to the Safety Officer, for his concurrence/approval before commencement of work.

Unless mentioned otherwise, all PERMIT shall be issued on a daily basis.

Requirement of different PERMIT, as spelt out by the Departmental Manager, shall have to be strictly adhered to after approval from the Safety Officer. Necessary assistance may be sought from the Safety Officer.

After erection of a specific scaffolding and subsequent approval by Safety Officer, any change in scaffolding structure– MUST BE BROUGHT UNDER NOTICE to safety officer by the competent supervisor deployed for the job.

      - Tool Box Talk:

In addition to above safety requirement all the workers including the supervisors have to undergo a tool box talk (about 10 minutes) before commencement of work in each day wearing specific PPE suitable for the job.

      - Personal Protective Equipment (PPE):

You shall ensure that all your staff and workmen have been issued appropriate PPE and they use the same at job sites. All these PPE shall conform to IS/CE/EN specification or other applicable international standards.

| Sl. No. | Work Areas | PPE Required |
| --- | --- | --- |
| 1 | All areas other than office areas/ ‘no PPE zone’ | Safety Shoes, Safety Helmet, General purpose Hand Gloves (Common PPEs). |
| 2 | Un-guarded elevated areas | Double lanyard full body safety harness, Fall Arrestor, Lifeline and any other job/ area specific PPE. |
| 3 | Confined space | High Visibility Fluorescent Jacket and any other job/ area specific PPE. |
| 4 | Public place/ road side | High Visibility Fluorescent Jacket and any other job/ area specific PPE. |
| 5 | Fabrication Yard | Material Handling Hand Gloves, Protective Eye wear and any other job/ area specific PPE. |
| 6 | Working on live lines | Protective Eyewear, FR Apron, FR Hand Gloves, Arm Guard (where applicable) and any other job/ area specific PPE. |
| 7 | Outdoor Switch/ Transformers Yard | Protective Eye wear, High Visibility Fluorescent Jacket and any other job/ area specific PPE. |
| 8 | Indoor Switch Room | Protective Eye wear and any other job/ area specific PPE. |
| 9 | Battery Room | Chemical Hand Gloves, Apron, Protective Eye wear, Nose Mask and any other job/ area specific PPE. |
| 10 | Pilot/ Cable Spreader Room | Protective Eye wear, High Visibility Fluorescent Jacket, Nose Mask, (where required) and any other job/ area specific PPE. |
| 11 | Cable Basement | Protective Eye wear, High Visibility Fluorescent Jacket, Nose Mask, Respirator & lifeline (if required) and any other job/ area specific PPE. |
| 12 | Power Generation Plant Area - TG Room, Boiler House, Compressor House, ESP areas, etc. | Protective Eye wear, Ear plugs/ muffs (as applicable) and any other job/area specific PPE. |
| 13 | Power Generation Plant Area - Coal Handling areas including Wagon Tippler, Track Hopper, Crusher House, etc. | Shin-Guard, Nose Mask, Protective Eye wear, High Visibility Fluorescent Jacket and any other job/ area specific PPE. |

You shall ensure that the above PPEs of approved standards are available at all times and shall be used by your employees with no exception whatsoever.

      - Safety Consequence Management Policy:

To improve safety in the workplace and to promote safe work practices, as per terms of the Safety Consequence Management Policy of the Company, while good performance in the workplace with regard to safety related behavior shall be rewarded, in the event of violations, unsafe behavior or deviations from the specified safety standards, rules, guidelines and relevant Regulations & Acts in force, are observed in activities performed by you or your sub-contractor and their employees, penal measures shall be imposed on you and concerned employees, which may be financial or suspension or both, as detailed hereunder, as per provisions of said policy in force:

| Penal Measures for Contractors for Safety violations |  |  |
| --- | --- | --- |
|  | Contractor |  |
| Incident | Owner | Supervisor & Workmen |
| Fatal | Rs. 2,00,000/- will be deducted for every incident. | Suspension of the concerned group for 30 days |
| LWDC (Lost Work Day Case) | Rs. 20,000/- will be deducted for every incident. | Suspension of the concerned group for 7 days |
| Violation of Safe Work Practices (for every site audit score < set cut off marks of 75%) | Rs. 5,000/- will be deducted for each audit | NA |
| Non-availability of required quality PPEs and safety gadgets during Gear / PPE Audit (for every audit score < set cut off marks of 90%) | Rs. 5,000/- will be deducted for each audit | NA |

The provisions of said policy in force may be ascertained from the respective dealing Officer of the Company. The assessment of the instances which may qualify for such reward or penalization shall be conducted by the empowered Officers of the Company, as laid down in the policy, and their decision shall be final and binding on you or their concerned employee, as applicable.

      - Training:

You shall not deploy any person at work place/ site without safety Induction Training. The safety card issued by the Company must be with the workmen all the time and same shall be produced on demand at any time.

The Safety Induction Course will require approximately at least one hour. The information provided during the orientation will include, but is not limited to such topics as:

      - Job rules, personal safety and conduct;

      - Hazard reporting;

      - Reporting of injuries/ pre-existing conditions;

      - Emergency procedures;

      - Working with Chemicals;

      - Safety Activities and Programme including disciplinary measure and incentives;

      - JHA/ SOP/ SMP/ SWP;

      - Job specific; and    Safety standards.

      - Behavioral base.

      - Skill development.

      - Safety Orientation Programme

You will submit the names of new employees to receive the orientation programme, at least two working days in advance prior to commencement of work. The Company will then schedule the orientation programme and advise You of the time and venue for the orientation programme.

You shall provide for and ensure that all employees must use appropriate Personal Protective Equipment (PPE) such as safety shoes, safety helmets, hand gloves, safety harness, safety goggles and all other safety appliances while working at site. You must be fully aware of the conditions of the work site and surroundings, such as presence of dust and gas, frequent movement of overhead equipment, rail & road traffic, simultaneous activities carried out by various agencies in a congested area etc. and shall plan and execute his work accordingly in a safe manner.

You shall also obtain a proper work permit/ clearance/ prior permission from the Operating Department before starting the work in a running plant/ pipe lines/ gas lines/ air lines/ water lines/ hazardous areas and for working on shut down days etc. You shall keep concerned authorities of the Company informed well in advance if the work is to be carried out on Sundays/ National Holidays/ Shut down days or at night. Safety Supervisor must talk and explain about job hazards daily before starting the work and shall adopt safety measures accordingly. You shall provide for and ensure the availability of relevant safety appliances as required for the work.

You shall carry out regular safety inspection at the work site to ensure that all safety measures are being properly maintained/ followed and maintain a record of observations made and corrective actions taken. You shall also carry out load test on his erection equipment, tools, tackles, lifting devices etc. and maintain a record of test results. You shall get all his equipment, tools and tackles examined by the competent authority as required by the Inspector of Factories. You shall produce required certificates for their fitness as and when asked by representatives of the Inspector of Factories.

You shall nominate a competent and authorized person, in writing to the Company, who shall

- Be fully responsible for safety of the employees; and

- Ensure safety practices during the execution of the Contract. This person shall be a designated person coordinating, administering and managing the safety effort on behalf of You. He shall be the focal point of liaison between the Company and you for all matters related to Safety at work sites. You shall also engage required number of supervisors and workers adequately trained to execute the work safely.

A proper register/ document containing names and designation of workers shall be maintained by you for safety trained workmen.